I S I E C

CERN

EP

INITIAL SAFETY INFORMATION on EXPERIMENTS at CERN

DATE: ____29.04.2003_________			EXPERIMENT:_CMS RPC

INSTALLATION START: __29.04.2003_		AREA/BEAM:  GIF X5C

SPOKESMAN: _________M. Della Negra

GLIMOS: ______C. Schaefer		TEL: ____160202__

FILLED IN BY: ___I. Crotty____		TEL: ____164414

1

TEST BEAMS: __________________GIF X5C

LABS AT CERN (Bldg/room): _________190 gas & exp. area

2

GASES, LIQUIDIS, CRYOLIQUIDS

Used in detectors (or kept in nearby containers):

| Device type | % Fluid 1 + % Fluid 2 + etc. | Volume | Abs. Press. | Max flow |
| --- | --- | --- | --- | --- |
| RPC | C2 H2 F4 | 57 +20l | 1.001bar | 70 l/hr |
|  | C4 H10 | 3 + 0.5l | 1.001 bar | 4 l/hr |
|  | SF6 | 0.2 +0.05l | 1.001 bar | 0.7l/hr |
| Storage (297) | C2 H2 F4 | 2 x 40l | 5 bar |  |
|  | C4 H10 | 2 x 40 | 3 bar |  |
|  | SF6 | 1 x 40 | 17 bar |  |

3

Other CHEMICALS

Toxic/corrosive/flammable metals, solvents, additives etc.:

____________________________________________________________________________________________NA______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

4

ELECTRICITY:

| MAGNETS: Magnet type | Power | Field | Gap vol. | Max water pressure |
| --- | --- | --- | --- | --- |

| Detector type | Voltage | Current | Stored energy | Nr. HV channels | Remote shut off |
| --- | --- | --- | --- | --- | --- |
| RPC | 10KV | 150A | 0.46J/channel | 37 | yes |
| Scintillators | 2KV |  |  | 3 | yes |

HIGH

VOLTAGE

(>1 kV)

SHORT-CIRCUIT currents > 5 mA for > 50 V possible anywhere? _____________

POWER dissipated by all electronics

          - On detectors: _____60W___

          - Off detectors: 6KW crates

SPECIAL GROUNDING requirements? ________________NO

5

LIFTING AND HANDLING:

Weight of heaviest single piece to install? ___650kg (Barrel)_50kg (endcap)

Specially designed handling equipment? _       NO____________________

For which max weights? _____________NA_________________________

6

VACUUM AND PRESSURE tanks (≠ 1 atm)

| Tank | Abs. Pressure | Volume | Weakest part(s) of the wall |
| --- | --- | --- | --- |
| NA |  |  |  |

7

RADIATION

Beam intensity, radioact. Sources depleted uranium, lasers, microwaves etc.:

____Source ; Cs 137 , 680 GBq and the Muon Beam

8

OTHER HAZARDS (or remarks):

________________________________NA________________________________________________________________________________________________________________________________________________________________________________________________________________________

COMMENTS		The ISIEC from 29.04.2003 filled in by A. Colaleo was updated today 21 Sept 2003 to take into account some small changes in Chamber configuration and nos. as well as the a set of scintillators.