- TO: 	CERN

- ATTENTION:  Mr. Krzysztof Doroba

DATE:  17/02/2005    N° OFFER:  99_05

- YOUR INQUIRY:  e-mail dtd. 05/02/2005

Dear Mr. Doroba,

please find our best quote according to your request:

CABLE  (2 X 2,5 + 2 X 0,22)HH2 + (2 X 2,5 + 2 X 0,22)HH2   -  IEC 60332-3-24  LS0H

Pairs of  0,22mm2

Tinned copper conductor 7 x 0,20mm

HDPE insulation   (nominal thickness 0,25mm   nominal diameter 1,10mm)

4 Conductors twisted in 2 pairs

Conductors of  2,50mm2

Bare copper conductor 48 x 0,25mm

Poliolefine insulation   (nominal thickness 0,60mm   nominal diameter 3,20mm)

Two groups 2x 2,5 + 2x0,22 individually shielded with Al/Polyester tape + tinned copper drain wire 7 x 0,20mm

Tinned copper braid   (wire diam. 0,10mm)

Groups laid up in round shape with LS0H fillers

Poliolefine outer sheath   (nominal thickness 1,40mm)

Nominal o.d. 16,65mm

Price x 1.400mt :   €/m 3,19

Packing : drum diam.1200mm x 1400mt length

Remarks: Price calculated  with copper at 2464 €/ton, copper content 143,45 kg/km. It will be fix within variation +/- 5%.

Fire test according IEC 60332-3-24: 500 €, cable required for the test  m  42.

Delivery:   6 weeks                                                                Carriage:   ex works

Packing:    drum included                                                      Quantity tolerance: +/-   5 %

Payment:  60 days                                                                  Offer validity at current costs: 40 days

Best Regards

novacavi  S.p.A.

Roberto Giudici