RPC Endcap	Gas Services	Station 1 (One Endcap)

- Chambers grouped in 20 and in series

| RPC | Nos Chamber Station | Nos. Gas Gaps /10 | Channel /chamber | Chambers /20 | Pipes /20 | Section /20 (cm2) 10mm | Total Nos. Pipes | Total Section /Endcap cm2 | Nos Pipes leaving Yoke | Section Leaving Yoke cm2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RE1/1 | 36 | 3 | 4 | 2 | 4 | 4 | 72 | 72 |  |  |
| RE1/2 | 36 | 3 | 4 | 2 | 4 | 4 | 72 | 72 |  |  |
| RE1/3 | 36 | 3 | 4 | 2 |  |  |  |  |  |  |
| Total |  | 9 |  | 6 | 8 | 8 | 144 | 144 |  |  |
| Total /Endcap | 108 | 324 |  |  | 216 | 216 | 144 | 144 | 7-9 | 24 |

# Pipes have an estimated diameter of 10mm down to the patch panel (Gas Rack) on the Yoke periphery.

## Supply pipes to Gas Rack		20 Flexible		x2 ( one for each height division)	Section 8cm2

## Return pipes to Gas Rack		20 Flexible		x2 ( one for each height division)	Section 8cm2

## Safety valve  exhaust to Gas Rack	20 Flexible		x1				Section 4cm2

## Back-up supply (power failure) to Gas Rackl		10 Flexible		x1		Section 1cm2

## Purge gas ( R134a)	 to Gas Rack				10 Flexible		x1		Section 1cm2

Analysis to Gas Rack						10 Flexible		x2		Section 2cm2

TOTAL											x9 pipes	Section 24cm2

Pipe section is 2 in cm2

IMC 09.2000