RPC Endcap	Gas Services	Station 4 (One Endcap)

- Chambers grouped in 20 and in series

| RPC | Nos Chamber Station | Nos. Gas Gaps | Channel /chamber | Chambers /20 | Pipes /20 | Section /20 (cm2) 12mm | Total Nos. Pipes | Total Section /Endcap cm2 | Nos Pipes leaving Yoke | Section Leaving Yoke cm2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RE4/1 | 18 | 2 | 4 | 1 | 4 |  |  |  |  |  |
| RE4/2 | 36 | 3 | 4 | 2 | 4 |  |  |  |  |  |
| RE4/3 | 36 | 3 | 4 | 2 | 4 |  |  |  |  |  |
| Total | 90 |  |  | 5 | 4 | 5.76 | 72 | 103.68 | 9 | 24 |

# Pipes have an estimated diameter of 8mm down to the patch panel on the Yoke periphery.

## Supply pipes to patch panel 		20 Flexible		x2 ( one for each height division)	Section 8cm2

## Return pipes to patch panel		20 Flexible		x2 ( one for each height division)	Section 8cm2

## Safety valve  exhaust to patch panel	20 Flexible		x1				Section 4cm2

## Back-up supply (power failure) to patch panel		10 Flexible		x1		Section 1cm2

## Purge gas ( R134a)	 to patch panel				10 Flexible		x1		Section 1cm2

Analysis to patch panel						10 Flexible		x2		Section 2cm2

TOTAL											x9 pipes	Section 24cm2

Pipe section is 2 in cm2

IMC 06.2000