Coaxial Cable							Ian. Crotty

3 Oct 2017

Small coal cable from CERN stores, hundreds of meters available.

C-50-1-1

### 04.61.11.A - COAXIAL CABLE 50 OHM - TYPE C-50-1-1

For any further technical information additional - click here

SPECIFICATION : LO-GE Nº461

CONDUCTOR        Copper

CONSTRUCTION     7 x 0.10 dia.                  dia : 0.30 mm

INSULATION       Solid XL-PE                    dia : 0.88 mm

SCREENING        Tinned copper braid 16x3x0.1mm dia : 1.33 mm

SHEATH           Brown polyolefine         max. dia : 2.10 mm

SPECIFICATIONS   CHARACTERISTIC IMPEDANCE       50 ± 2 Ohm

RATED CAPACITANCE              101 pF/m

PROPAGATION RATE               0.66

RATED DELAY                    5.03 ns/m

MAX. ATTENUATION           MHz 10 100 200

dB/100 m 15  46  66

DC RES. OF INNER CONDUCTOR     323 mOhm/m

DC RES. OF SCREEN              53 mOhm/m

MAX. RATED VOLTAGE             1.2 kV r.m.s.

TEST VOLTAGE PART. DISCH.<1pC  1.3 kV r.m.s.

DIELECTRIC STRENGTH TEST VOLT. 3 kV d.c.

OPERATING TEMPERATURE          -10 to +70°C

ANTI-FIRE TEST                 IEC 332-1

ACCORDING TO SAFETY INSTRUCTION IS 23

CERN Stores SCEM 04.61.11.105.3

Cost	0.88chf/m

https://edh.cern.ch/edhcat/Browser?command=showPage&argument=719&top=719&objid=%24%24EDH75gc96hf3&showAdvanced=&scem=&keywords=

OR

C-50-1-1A

### 04.61.11.AD - COAXIAL CABLE 50 OHM - TYPE C-50-1-1A

For any further technical information additional - click here

SPECIFICATION : LO-GE Nº515

CONDUCTOR        Silver-plated copper

CONSTRUCTION     7 x 0.10 mm dia.              dia. 0.30 mm

INSULATION       SILTEM                        dia. 1.10 mm

SCREENING        Silver-plated copper braid    dia. 1.55 mm

16 x 3 x 0.10 mm

SHEATH           Grey SILTEM

SPECIFICATIONS   CHARACTERITIC IMPEDANCE       50 ± 2 Ohm

RATED CAPACITANCE             120 pF/m

PROPAGATION RATE              0.66

RATED DELAY                   5.03 ns/m

MAX. ATTENUATION          MHz 10 100 200 400

dB/100 m -- --  --  113

SWITCH-OFF FREQUENCY          70 GHz

DC RES. OF INNER CONDUCTOR    300 mOhm/m

DC RES. OF SCREEN             53 mOhm/m

MAX. RATED VOLTAGE            2 kV r.m.s

TEST VOLTAGE PART. DISCH.<1pC 1 kV r.m.s

DIELECTRIC STRENGTH TEST VOLT.3 kV r.m.s

OPERATING TEMPERATURE         -40 to +135ºC

ANTI-FIRE TEST                IEC 332-1

CERN Stores SCEM 04.61.11.090.3

Cost 3.6chf/m

https://edh.cern.ch/edhcat/Browser?command=showPage&argument=721&top=721&objid=%24%24EDH75gc96hf3&showAdvanced=&scem=&keywords=

From outside CERN, Draka which is the same as the C-50-1-1 available from CERN.

http://prsmn.com/en/business_markets/markets/multimedia/downloads/datasheets/hf31e.pdf