List of comments for the PK chambers delivered in 3 June 05

- Hv connector

- Brazing of coo pipe

- 0V cable section  0.75 – 2.0mm

- PVC pipes gas plugs

- Shiels dbox secured at edge a and not in the 6 holes provided

- Gas Pipe height ??

- HV cables and signal htreaded thru’ each other

- Solder blobs

- Flat cable not cut off short enough

- Solder top and bot Cu F. shield together

- Double stick for ditto—insulating the two

- Cu cleaned but what of flux , what type—corrosion

- Good thick white Mylar 200µm

- Top gap very bent >10mm

- Loose unions

- HCP milled out leaving very rough surfaces

- No protection of cut-out edges (tape)

- Spacer test Open gap

- heights of chamber		56.4mm- 55.4

- Heights of shield 25

- Heights of chamber frame 31.5-32mm

- Insulation of sig ref to chamber structure

- Cu cooling is rough and design is complicated

- Where are the travelers (data )

- Pipe cross over

- Packing was horizontal and not vertical

- Integration gives 97mm so is too high for CSC alignment sensors

- Alu covers for the coaxial cables.

Ian Crotty  060605