Stages and Action to ready RE1/1 chambers for operation in CMS

Stages

- Gas Gap Testing

- Internal chamber Assembly

- External assembly and QC inspection

- Cosmic tests

- Transport and Installation in SX5 on YE-1

- Service installation, inc temp. Signal cables to Tower racks

- Commissioning

Tasks

- Gas Gap Testing {leak and HV)

    - 20 mbar

    - Longest duration HV test possible.

- Internal Chamber assembly

    - Gas pipe hot forming

    - Check shielding insulation

    - Shielding integrity

    - Solution to obtain parallel flow ?

- External Chamber Assembly

    - Signal extraction change to 2mm coaxial, trials with ferrules ?

    - Flat cable and bulkhead connexions.

    - Cooling unions to inch pipe.

    - FEB and LV dist installation with LV cables.

    - HV connexions.

    - Ground cables and single point.

    - External gas distribution and impedance.

    - Install temperature sensors on these cramped chambers.  Do we use a connector on the Chambers bulkhead ?

    - Make trial run insertion of 2 chambers to look for interferences with each other and installation questions.

    - Check cooling circuit for leaks at 15 bar.

- Cosmic tests

    - Do continuity and insulation tests

    - Threshold check

    - Signal, dead strips, connector inversion et cetera

    - Dark current and change over time.

- Transport & Installation

    - Use original travel box

    - Inform ME1/1 colleagues .

    - Require one cherry picker and >2 people.

    - Use hand insertion, no crane.

    - Install temporary signal cable and DCS from RE1/1 position to towers through chock points.

    - Connect to the cooling system removing present short circuit.

    - Make gas connexions using metal or poly piping.

    - Reroute FO somewhere ?

    - Identify all services

- Commissioning

    - Gas flow test

    - HV and dark current

    - Cooling pressure test and labels

    - Install additional Link Boards

    - Check FO output

Ian Crotty	07 Dec 07