Draft Checklist on RE1/2 Assembly Procedure

| Step No: | Check item | Status (ready?) |
| --- | --- | --- |
| A01 | Check if all components are ready |  |
| A02 | Record SN of gas-gaps, mechanics box |  |
| A03 | Put bottom HCP on the assembly table |  |
| A04 | Fix insulation mylar on HCP, trim HV slot |  |
| A05 | Mount 3 L-fixation brackets on non-HV side (not tight) |  |
| A06 | Fix bottom copper with mylar on HCP, cut HV slot |  |
| A07 | Put bottom gas-gap |  |
| A08 | Trim copper edge at the gap corner and HV position |  |
| A09 | Put 3 soldering protection strip on gap |  |
| A10 | Put readout strip plane |  |
| A11 | Put two top cut gaps on strip plane |  |
| A12 | Put two top cut copper with mylar on top gaps, cut HV slots on copper and mylar |  |
| A13 | Put 6 glass gap fixation pieces in position |  |
| A14 | Mount the Al bar at the narrow end |  |
| A15 | Mount 3 L-fixation brackets on HV side (not tight) |  |
| A16 | Adjust gaps, strip and copper position |  |
| A17 | Tighten the 6 L-fixation brackets on both edges |  |
| A18 | Connect the gas-pipe of the top cut gaps together |  |
| A19 | Bend the bottom copper onto top copper, glue them together with conductive copper tape, solder several points at gap free area |  |
| A20 | Solder copper ground wire at gap free area |  |
| A21p | Put braid screen copper onto HV and ground cables and make HV heads |  |
| A22p | Tape the two long gas pipes in position |  |
| A23p | Put 3 3 soldering protection strip between top copper and mylar |  |
| A24p | Solder co-ax cable onto strips and copper |  |
| A25p | Make gas connections to the top Al bar, and record the correspondence between gas outlets and gaps |  |
| A26p | Fix insulation mylar on top HCP, trim HV slots, tape the cable extraction slot edges |  |
| A27p | Fix 6 flat cables onto patch panel |  |
| A28p | Fix cooling pipes onto the patch panel |  |
| A29p | Fix FEB on cooling supporting plates |  |
| A29 | Check conductivity, insulation between strips, copper ground and adaptor board |  |
| A30 | Mount the side Al bars |  |
| A31 | Mount the top HCP |  |
| A32 | Mount patch panel (with flat cables and cooling system |  |
| A33 | Fix cooling pipes onto top HCP |  |
| A34 | Mount HV head onto patch panel |  |
| A35 | Connect FEB with flat cables and adaptor boards |  |
| A36 | Tape down flat cable and arrange them in position |  |
| A36 | Mount distribution board and collect it with flat cable |  |
| A37 | Mount screen box |  |
| A38 | Mount corner L-brackets (for CSC joint) |  |

Note: A**p means these steps could be done in parallel.