Draft Checklist on RE1/2 Assembly Procedure

| Step No: | Check item | Status (ready?) | Operator | Date |
| --- | --- | --- | --- | --- |
| A01 | Check if all components are ready |  |  |  |
| A02 | Record SN of gas-gaps and mechanics box |  |  |  |
| A03 | Put bottom HCP on the assembly table |  |  |  |
| A04 | Fix insulation mylar on HCP, trim HV slot |  |  |  |
| A05 | Mount 3 L-fixation brackets on non-HV side (not tight) |  |  |  |
| A06 | Fix bottom copper with mylar on HCP, cut HV slot |  |  |  |
| A07 | Put bottom gas-gap |  |  |  |
| A08 | Trim copper edge at the gap corner and HV position |  |  |  |
| A09 | Put 3 soldering protection strip on gap |  |  |  |
| A10 | Put readout strip plane |  |  |  |
| A11 | Put two top cut gaps on strip plane |  |  |  |
| A12 | Put two top cut copper with mylar on top gaps, cut HV slots on copper and mylar |  |  |  |
| A13 | Put 6 glass gap fixation pieces in position |  |  |  |
| A14 | Mount the Al bar at the narrow end |  |  |  |
| A15 | Mount 3 L-fixation brackets on HV side (not tight) |  |  |  |
| A16 | Adjust gaps, strip and copper position |  |  |  |
| A17 | Tighten the 6 L-fixation brackets on both edges |  |  |  |
| A18 | Connect the gas-pipe of the top cut gaps together |  |  |  |
| A19 | Bend the bottom copper onto top copper, glue them together with conductive copper tape, solder several points at gap free area |  |  |  |
| A20 | Solder copper ground wire at gap free area |  |  |  |
| A21p | Put screen copper braid onto HV and ground cables, make HV heads (small metal plug) for each gaps (may already be made during gap leak and HV test) |  |  |  |
| A22p | Tape the two long gas pipes in position |  |  |  |
| A23p | Put 3 soldering protection strip between top copper and mylar |  |  |  |
| A24p | Solder co-ax cable onto strips and copper |  |  |  |
| A25p | Make gas connections to the top Al bar, and record the correspondence between gas outlets and gaps |  |  |  |
| A26p | Fix insulation mylar on top HCP, trim HV slots, tape the cable extraction slot edges |  |  |  |
| A27p | Fix 6 flat cables onto patch panel |  |  |  |
| A28p | Fix cooling pipes onto the patch panel |  |  |  |
| A29p | Fix FEB on cooling supporting plates |  |  |  |
| A30 | Check conductivity, insulation between strips, copper ground and adaptor board |  |  |  |
| A31 | Mount the side Al bars |  |  |  |
| A32 | Mount the top HCP |  |  |  |
| A33 | Mount patch panel (with flat cables and cooling system |  |  |  |
| A34 | Fix cooling pipes onto top HCP |  |  |  |
| A35 | Connect FEB with flat cables and adaptor boards |  |  |  |
| A36 | Tape down flat cable and arrange them in position |  |  |  |
| A37 | Mount distribution board and connect it with the flat cable |  |  |  |
| A38 | Mount screen box |  |  |  |
| A39 | Put the chamber into cosmic test rig |  |  |  |
| A40 | Flow the RPC chamber with working gas until 20 gas volume exchange |  |  |  |
| A41 | Connect three ground cables from gaps together and to common ground, check copper ground and all other grounds are connected properly |  |  |  |
| A42 | Connect HV cables from each gas-gap to HV module and apply HV |  |  |  |
| A43 | Perform a quick cosmic test |  |  |  |
| A44 | move the chamber out ot the cosmic rig |  |  |  |
| A45 | (If test results positive) cut the small metal plug at each HV cable, make final HV head onto the three HV cables (two top cut gaps joint) |  |  |  |
| A46 | Mount HV head onto patch panel |  |  |  |
| A47 | Mount screen box, cover all edge and slots with conductive Al tape |  |  |  |
| A48 | Mount corner L-brackets (for CSC joint) |  |  |  |
| A49 | Perform a thorough cosmic test |  |  |  |

Note: A**p means these steps could be done in parallel.