CMS RPC Endcap Cable Connectorising

Request for quote

Christian Dechelette tel 74337/160043 (WS Tel; 74771)

- First stage

This action is to cover the initial installation to be done before ‘Offyoke’ ME1/2 & RE1/2 chambers are installed

RE1/1 ONE Endcap

| Type | Calc. Quantity | Quantity Nos | Approx Length Range [m] | Number of Pins per Connector | Connector Ref. | Cable Ref. | Remarks | Status | Requested Delivery Date. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LV RPC | 36 | 18 | 10 - 25 | 8 x 0.75 | Molex 43025 | Novacavi FMOH2M1 | 1 cable for 2 chambers |  | Jan 05 |
| LV LBB Power | 6 | 36 | 10-25 | 4 |  | 04. |  |  | Jan 05 |
| LV LBX Sense |  | 36 | 10-25 | 4 |  | 04 |  |  |  |
| HV | 36 | 36 | 10-16 | 3 x 0.22 [Cable 4x0.22] | CPE 28.002.512_020 | Novacavi P0825_03 |  |  | Jan 05 |
| Trigger | 36 x 8 | 288 | 6 | 40 AWG 28 | 09.55.03.340.4 | 04.71.06.400.0 | Not Laminated |  | Jan 05 |
| DCS RPC | 36 | 36 |  | 24 ? |  |  |  |  | Jan 05 |
| DCS LBB |  |  |  |  |  |  |  |  | Jan 05 |
| Temp. Sensor | 36 | 36 |  | 2x0.22mm2 | 09.55.03.310.0 | 04.21.51.055.4 |  |  | Jan 05 |
| Optical fibres | 6 x 2 x 2 | 24 |  |  |  |  |  |  | Jan 05 |

- Second Stage

RE1/2 & RE1/3 ONE Endcap

| Type | Calc. Quantity | Quantity Nos | Approx Length Range [m] | Number of Pins per Connector | Connector Ref. | Cable Ref. | Remarks | Requested Delivery Date. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LV RPC | 36 x 2 | 72 | 9 - 18 | 8 x 0.75 | Molex 43025 | Novacavi FMOH2M1 |  | Feb 05 |
| HV | 36 x 2 | 72 | 10-23 | 3 x 0.22 [Cable 4x0.22] | CPE 28.002.512_020 | Novacavi P0825_03 |  | Feb 05 |
| Trigger | 36 x 6 x 2 | 432 | 6-13 | 40 AWG 28 | 09.55.03.340.4 | 04.71.06.400.0 | Not laminated | Feb 05 |
| DCS RPC | 36 x 2 | 72 |  | 24 ? |  |  |  | Feb 05 |
| Temp. Sensor | 36/6 x 2 | 36 |  | 2x0.22mm2 | 09.55.03.310.0 | 04.21.51.055.4 |  | Feb 05 |

- Third stage

RE2 and RE3 One Endcap

| Type | Calc. Quantity | Quantity Nos | Approx Length Range [m] | Number of Pins per Connector | Connector Ref. | Cable Ref. | Remarks | Requested Delivery Date. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LV RPC | 36 x 2 x 2 | 144 | 5 - 15 | 8 x 0.75 | Molex 43025 | Novacavi FMOH2M1 |  |  |
| HV | 36 x 2 x 2 | 144 |  | 3 x 0.22 [Cable 4x0.22] | CPE 28.002.512_020 | Novacavi P0825_03 |  |  |
| Trigger | 36 x 6 x 2 x 2 | 864 |  | 40 AWG 28 | 09.55.03.340.4 | 04.71.06.400.0 | Not laminated |  |
| DCS RPC | 36 x 2 x 2 | 144 |  | 24 ? |  |  |  |  |
| Temp. Sensor | 36/6 x 2 x 2 | 72 |  | 2x0.22mm2 | 09.55.03.310.0 | 04.21.51.055.4 |  |  |

Ian Crotty  4 Nov. 04