Signal Cable assembly for RE2/2

2 pages , 1 one for each end of the cable

Note the cable length calculation for RE1/3 is NOT the same.

Link Board Box End

Enlever la gaine L= 400mm

40 pin Scotch Flex (3M)

Cosse diam 6 x 10mm

Ground wire 0.5mm2

L= 300mm

L= 40mm

Heat Shrink Tube

Diam 10

With internal Hot melt

Heat Shrink Tube

Diam 12 L = 60mm

NE PAS Chaufer.

SCEM 04.86.62.130.9

Chamber End

Enlever la gaine L= 550mm

40 pin Scotch Flex (3M)

Cosse diam 6 x 10mm

Ground wire 0.5mm2

L= 250mm

L= 40mm

Heat Shrink Tube

Diam 10

With internal Hot melt

Ian Crotty

23 April 06