Mock-up RE3/1										Ian Crotty

14 Oct 2016

Use thin Alu sheet 0.5 – 0.7mm

Not in CERN stores

Contact Leila Akhouay 72239

Use @U@ channel section 25 x 15mm

44.02.31.025.2

Sheets

Trapezoid

638mm 	small base

1244mm	large base

1717mm	Height rediuce height by ~100 mm should bring clearances where necessary

Gives approx.		1600mm	Height

1244mm 	Large base

638mm 	Small base

Weight of o.5mm sheet			1.35 [kg/m2]

Area Trapezoid		=	(Lb+ Sb)/2 x H

=	1.51 [m2]

Weight 0.5mm		=	4.1kg

Weight 1.0mm		=	8.2kg

Channel section 25 x 15 x 2mm

Weight = 0.278kg/m

Perimeter			1.6m x 2 + 1.24m + 0.64m	=`	5.1m

Weight 	= 	1.42kg

Totak weight 		=	1.42 + 8.2	=	10kg